Contents

| Assignment Number | Page Number |
| --- | --- |
| 1 | 2 |
| 2 | 15 |
| 3 | 17 |
| 4 | 19 |
| 5 | 21 |
| 6 | 22 |
| 7 | 23 |
| 8 | 25 |
| 9 | 27 |
| 10 | 31 |
| 11 | 32 |
| 12 | 34 |
| 13 | 37 |
| 14 | 40 |
| 15 | 43 |
| 16 | 45 |
| 17 | 46 |
| 18 | 48 |
| 19 | 50 |
| 20 | 51 |

1.	You are in charge of the Survey Team of the Road Transport Authority of your 	country. You have been asked to conduct a survey for a proposed road project of 	150 km long and 100 m wide. Write a short Feasibility Study Report which 	should consist of the following:

- Types of surveying to be conducted.

- Types of surveying instruments and accessories to be used.

- Necessary measurements to be carried out.

- Necessary computation to be made.

- Necessary drawings to be prepared.

- Quantities of earthwork (cutting and filling) to be computed.

- Obstructions and difficulties to be anticipated and their probable remedies.

- References.

- Types of surveying conducted

Since the surface area to be covered by the proposed road is approximately 50 km by 100 m, that is about 5 km2, therefore plane surveying will be adequate.

Plane surveying can be divided into the following:

#### Traverse surveying

#### Linear surveying

- Ordinary leveling

First, an open traverse survey will be conducted. A diagram of an open traverse is shown below:

N	Starting station of the road

Ending station of the road

The length and bearing of the starting line are measured and by computing the latitude and departure for each of the lines, the position of each station and length of each line will be known.

Ordinary leveling must also be conducted to determine the various levels of each of the stations on the traverse.

In addition, stadia surveying, otherwise known as tacheometry surveying, will be conducted along various points on the road to determine the vertical and horizontal distance of the road so that the contour of the road can be determined.

Last but not least, profile leveling will be conducted in the case of road surveying. Profile leveling is a specialized form of differential leveling in which elevations are taken at specific points along the intended route of a highway. The purpose is to establish the profile and the cross-section along the right-of way.

b)	Types of surveying instruments and accessories used

- Distances are measured with the Electronic Distance Measurement (EDM) 	equipment. An EDM instrument provides quite accurate distance readings 	in the blink of an eye. It determines the round-trip distance between two 	points in terms of an exact number if wavelengths of modulated 	electromagnetic energy. An advantage is that it can be operated in haze or 	light rain.

An EDM reflector is used together with an EDM to reflect the emitted               	electromagnetic wave back to the EDM for distance calculation.

- Horizontal and vertical angles are measured with a theodolite. It is an instrument that make it possible to read angles to very small fractions of a degree, thus making the surveying results very accurate.

- A level is used to determine the height of various stations in the surveyed area. It is a tripod mounted telescope whose optical axis is perpendicular to a vertical reference, the latter being a leveling staff. It contains a mechanism for bringing the telescope axis to horizontal and permits rotation about the vertical axis so that the line of sight may be pointed in the desired direction. The staff and the telescope axis are maintained parallel to one another so that the act of bringing the level bubble to center also brings the telescope optical axis to the horizontal. The bubble will looked like the following when leveled:

Not leveled				         Leveled

- A leveling staff will be used together with a level. This staff is 3 metres long and is calibrated at equal intervals for the purpose of measuring vertical heights of various points. In order to hold the staff as vertical as possible, a leveling bubble is used.

Not leveled			   	         Leveled

The bubble will look like the figure on the right when the staff is vertical.

v)	A theodolite is mounted on a tripod and a plumbline is hung from the tripod. Through the theodolite, the tripod can be adjusted so that the theodolite can be shifted to above the exact location of the station. Also, bubbles available on the theodolite need to be adjusted so that the theodolite is horizontally leveled. An example of the theodolite bubble is shown below:

Not leveled

Not leveled

Leveled (Bubble at rest in the center)

vi)	Lastly, a prismatic compass will be used to measure bearings of assigned points on the road. In this case, it will be employed in the measurement of the bearing of the starting station of the road. It consists of a magnetic compass mounted on a tripod.

c)	Necessary measurements to be carried out

- Using an EDM, with the instrument placed at a station and the EDM 	reflector placed at another station, the distance between the 2 instruments 	will be obtained as shown below :

d



H

Since the angle of elevation or depression can be obtained using a 	theodolite, the exact horizontal distance between 2 stations can be 	calculated as shown,

H = d cos 

To minimize random errors, 2 readings will be taken, one on each face, 	that is one left face reading and one right face reading. Subsequently, an 	average of the 2 readings will be taken.

- To determine the vertical distance between 2 stations, a tripod-mounted telescope known as a level is used. Through the level, we will be able to see a circular screen with lines as shown:

164

162

wedge

leveling staff

To read the readings from a leveling staff, the circular screen will be aligned to the reading of the staff as shown above. In this figure, the reading will be taken as 163.

- The distances and heights around each station can be obtained using a theodolite. By placing the level staff at various points on the survey ground, and the theodolite adjusted to a certain vertical angle, the approximate level of these points can be calculated from the vertical angle and the staff level measured using the derived formulas. These values will be needed to construct a topographic map or plan with contour lines.

- In tacheometry, the various levels of selected points on the road will be measured using a theodolite and a leveling staff. The setup for this process is shown below:

Three readings will be taken from the staff, namely the upper, middle and lower readings.

d)	Necessary computation to be made

i)	From the prismatic compass, we can measure the bearing of the starting station. Also, using the theodolite and the EDM, the respective angles and distances between each of the stations are measured and recorded.

ii)	When all the angles are measured, they are tabulated so that necessary calculations can be carried out and recorded. The necessary calculations will be needed to compute the following information:

- The whole circle bearings and reduced bearings of all the stations will be calculated.

- With the distance and angles between 2 stations known, the latitude and departure of one station relative to another can be computed and eventually, they will be recorded.

- When the latitudes and departures are known, coordinates of all stations with respect to one point can be determined.

All these values will be tabulated onto a traverse computation chart, and any random errors detected from the calculations will be accounted for and recorded in the chart.

- The height of each station will be measured using a level and a staff.

Given also the benchmark, the various reduced levels of each station can 	be computed easily. In addition, data like height of instruments and height 	of collimation will be calculated and recorded. In the process of 	determining the height of each station, staff readings such as back sight, 	fore sight and intermediate sight will be measured and recorded. If the 	terrain situation makes it impossible to measure the height of one station 	with respect to the other directly, then a Change Point may be introduced 	to solve the problem.

iv)	A table will be drafted to record the readings obtained, that is, the back sight, fore sight and the intermediate sight. Depending on the method employed in obtaining the reduced levels of each point, which is either the rise and fall method or the height of collimation method, a table for one of the method will be constructed for data entry and calculations.

- Lastly, from tacheometry surveying, three readings will be taken from the staff at each of the selected features or points on the road. From these readings, we will be able to determine the horizontal and vertical distances of that selected point with respect to a station using either two derived formulas or the table provided.

The 2 formulae are

H = 100 s cos2

V = 50 s sin2

where H is the horizontal distance,

V is the vertical distance,

 is the vertical angle,

s is the intercept ( top stadia reading - bottom stadia reading )

when all the distances and angles are measured and recorded, they will be tabulated.

- Necessary drawings to be prepared

Based on the information obtained, a topographic map will be drawn. The topographic map will display the following information:

- Contours.  The approximate levels of various points on the surveyed area will be collated and contour lines will be drawn for every fixed contour interval, for example a constant vertical interval of one metre.

Closely spaced contours will represent steep slopes while broadly spaced 	contours indicate gentle slopes. Changing contour spacing shows changing 	slope; thus unevenly spaced contours indicated rugged or undulating 	terrain while evenly spaced contours indicate a constant surface gradient.

A contour line is continuous and will close on itself within a map 	boundary or else originate at one point on the map boundary and pass off 	the map at another point along the boundary.

Contours do not cross one another except for the case where there is an existence of an overhanging cliff.

- Specific features. Specific features such as buildings, rivers, footpaths, trees, bushes, edges of cliffs, summit of a round hill, shoreline, banks of rivers, drainage paths on the flanks of a tall cliff and round hill, ridge lines between drainage paths and low points.

The topographic plan should be drawn to a suitable scale such that it can cover the contour and special features precisely. A rough topographic plan may also be sketched to illustrate general information besides details. The figure below illustrate an example of a topographic plan that should be presented:

After open traverse is conducted, the plan as shown above is drawn. Various stations that connect the traverse will be marked at the approximate roadside. At approximately every 10 m interval from the start point to the end point, profile sections, that is cross sections, will be drawn at the bottom of the plan as shown.

The topographic map provides an idea of the terrain in the surveyed area. It will also display how the elevation varies with distances from various points. In general, the following useful information can be extracted from the map:

- We can determine the number of trees and bushes in the surveyed area and thus, able to know the exact number of trees to be cut and bushes to be cleared to make way for the proposed road.

- The approximate levels of various points on the map are known. Therefore we are able to determine the amount of soil to cut and the amount of land to be filled in order to construct a relatively even road without severe height variations in different portions of the road.

- Certain fixed buildings and objects along the proposed route may also be noted and the turning and meandering route of the proposed road may be planned in advanced.

- The kind of terrain that can be found on the surveyed area will be known, such as a grass patch, a barren land.

- The profile of the centre line of the road will also tell us how the cross-section will look like. Knowing that, we can roughly visualize the nature of the road and determine how much cutting and filling of the land has to be done so that the road will not be slanted sideways.

- Rivers and streams, if there exists, will also be displayed on the map. From these features on the map, we can determine the number of rivers to be bridged, length of the bridge and how it should be bridged in order to form the road.

- Finally, from the map, we will find out how many buildings are built around the proposed road and how the future road construction will affect them. Thus, we can offer an acceptable sum of compensation to thee buildings' owners to make up for the negative effects that the road building might cause.

To conclude, I should say that the map is useful in providing us with the necessary information on the major obstacles that are likely to arise from the road construction, thus enabling is to plan ahead.

f)	Quantities of earthwork (cutting and filling) to be computed

From the elevation profile drawn, one can determine where to cut and fill. To compute the cut or fill, the reduced levels at each distance point must be found. The proposed level of the new works at each distance point is calculated. The cut or fill is the difference between the proposed level and the reduced level each point. A cut is required when the surface level is higher than the proposed level and a fill is needed when the surface level is lower than the proposed level.

The cross section across the width of the road must also be calculated so that the total volume of cut or fill can be calculated.

g)	Obstructions and difficulties to be anticipated and their probable remedies

In surveying, the survey team may find obstructions and difficulties that make the construction of the original proposed route unrealistic. It is important to find alternatives and solutions to the problems before the actual construction starts. Otherwise, the whole project may fail. Below it an outline of the possible problems that may occur.

- Residential areas lying on the path of the proposed road. Paying compensations to them can do resettlement of the residents. If it is not economical to pay the large compensations or if the residents refuse to move, it would be a better idea to detour.

- Religious institutions lying on the path on the proposed road. Religion is a very sensitive issue. If the religious institutions are deserted and dilapidated, they may be demolished. However, if the residents have strong faith in them, it is better to detour, to avoid resentment.

- Hills or mountains lying on the path of the proposed road. The developers can choose to detour, use explosives to blow up the mountain or dig tunnels through to mountain.

- Large rivers stretching across the path of the proposed road. As filing up the river would upset the ecosystem, a suitable solution is to build a bridge across it.

- Deforestation may upset the ecological system. Deforestation is inevitable if the road have to pass through thick forests. Reforestation should be carried out to complement the trees being chopped off once the road is completed.

- Pollution. Pollution should be kept to minimum. Chemical waste should be disposed properly, rather than to dump them into rivers.

##### h)	Conclusion

Surveying is essential to all civil engineering projects and it is important to avoid misconceptions of the traverse or any unforeseen problems that may hinder the construction project. All maps have to be carefully plotted and all obstacles analyzed and possible solutions work out. Otherwise, wrong decisions may lead to loss in precious time, money or even lives.

- References

- “Class notes”

By Dr M.A. Aziz (NUS, Singapore)

- “Surveying For Construction”, fourth Edition

By William Irvine (McGram, Singapore)

2.	A, B and C are three shore stations on a coastline and P is a sounding point at sea 	as shown in Figure 2. The following measurements were taken by an EDM and a 	theodolite.

AB	=	400 m

BC	=	381 m

ABC	=	1223000

BAP	=	720245

BCP	=	702715

A and C are respectively east and west of BP. P and B are respectively north and south of AC. Calculate the distances AP, BP and CP.

## P

N

O

## C

## A

Figure 2

B

Consider ABC, by Cosine Rule,

AC2	=	3812 + 4002 – 2(381)(400)cos(1223000)

AC	=	684.78 m

Consider ABC, by Cosine Rule,

381 / sinBAC	=	684.78 / sin(1223000)

 	BAC		=	27597.21

Consider OAB,

OA / 400	=	cos (27597.21)

	OA	=	353.23m

Consider OAP,

CAP		=	720245 - 27597.21

=	440337.79

353.23 / AP	=	cos (440337.79)

	AP	= 	491.54 m

Consider ABP, by Cosine Rule,

PB2	=	491.542 + 4002 – 2(491.54)(400) cos (720245)

BP	=	529.53 m

Consider ABC,

ACB		=	180 - 27597.21 - 1223000

=	293052.79

Consider ACP,

ACP		=	702715 - 293052.79

=	405622.21

By Sine Rule,

491.54	/ sin (405622.21)	=	CP / sin (440337.05)

	CP			=	521.67 m

- To find out the distance between two inaccessible points C and D, as shown in Figure 3, two stations were selected 100 m apart. The following observations were made by a theodolite.

CAD	=	453024		DAB	=	573012

CBA	=	561036	and	CBD	=	502006

Calculate the distance CD.

D

C

O

Figure 3

## A

B

Consider OAB,

AOB		=	COD

=	180 - 573012 - 561036

=	661912

DOB		=	AOC

=	180 - 661912

=	1134048

Consider AOB, by Sine Rule,

OB / sin (573012)	=	100 / sin (661912)

	OB		=	92.10 m

OA / sin (561036)	=	100 / sin (661912)

	OA		=	90.71 m

Consider OBD,

ODB		=	180 - 502006 - 1134048

=	155906

By Sine Rule,

OD / sin (502006)	=	92.10 / sin (155906)

	OD		=	257.44 m

Consider OAC,

OCA			=	180 - 453024 - 1134048

=	204848

OC / sin (453024)	=	90.72 / sin (204848)

	OC		=	182.11 m

Consider COD, by Cosine Rule,

CD2	=	182.112 + 257.442 – 2(182.11)(257.44) cos (66o1912)

CD	=	248.56 m

- The whole circle bearings of lines AB and BC are 1824 and 5054 respectively. The coordinates of points A and C are as follows:

| Point | Northings | Eastings |
| --- | --- | --- |
| A | 300.00 m | 400.00 m |
| C | 1430.90 m | 1250.80 m |

Find the lengths of lines AB and BC.

Draw a neat sketch.

B2

C (1250.80, 1430.90)

5054

A2

B

B1

1824

A (400,300)                                        	         	         C1

A1

C1C		=	1430.90 – 300.00

=	1130.90 m

AC1		=	1250.80 – 400.00

=	850.80 m

AC		=	 [(AC1)2 + (C1C)2]

=	 [(850.80)2 + (1130.90)2]

=	1415.20 m

CAC1	=	tan-1 [(CC1)/(AC1)]

=	tan-1 [1130.90/850.80]

=	530242

BAC		=	9000  A2AB  CAC1

=	9000  1824  530242

=	183318

ABB1	=	A2AB

=	1824

B1BC	=	180  B2BC

=	180  5054

=	12906

ABC		=	ABB1 +  B1BC

=	1824 + 12906

=	14730

BCA		=	180  BAC  ABC

=	180  183318  14730

=	135642

Consider triangle ABC,

AC	      	=	           AB	    	=	              BC

sin (ABC)			  sin (BCA)			        sin (BAC)

1900.52         	=	           AB	    	=	              BC

sin (14730)		  sin (135642)		      sin (183318)

AB	=	634.75 m

BC	=	838.15 m

- Table below shows the partial coordinates of a closed traverse. Calculate the following:

a.	Closing error.

b.	Accuracy of the traverse.

c.	Corrected coordinates of all stations, given that station P is the origin.

| Line | Distance (m) | Easting | Westing | Northing | Southing |
| --- | --- | --- | --- | --- | --- |
| PQ | 122 | 55.40 | - | 108.70 | - |
| QR | 156 | 73.20 | - | - | 137.70 |
| RS | 65 | - | 20.10 | - | 61.80 |
| ST | 60 | - | 56.40 | - | 20.50 |
| TP | 123 | - | 52.90 | 110.50 | - |

| Line | Distance (m) | Easting (+) | Westing (-) | Northing (+) | Southing (-) |
| --- | --- | --- | --- | --- | --- |
| PQ | 122 | 55.40 | - | 108.70 | - |
| QR | 156 | 73.20 | - | - | 137.70 |
| RS | 65 | - | 20.10 | - | 61.80 |
| ST | 60 | - | 56.40 | - | 20.50 |
| TP | 123 | - | 52.90 | 110.50 | - |
|  | 526 | 128.60 | -129.40 | 219.20 | -220.00 |
|  | ### Error | D= -0.80 |  | L= -0.80 |  |

a)	Closing error	= [(D)2 + (L)2] ½

= 1.131 m

b)	Perimeter	= 122 + 156 + 65 + 60 + 123

= 526 m

#### Accuracy 	= Closing Error / Perimeter

=  2.151 x 10-3

c)	Correction to latitude and departure by Bowditch Rule

| Line | Correction to latitude | Corrected latitude | Correction to departure | Corrected departure | Station | Coordinates |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Northings | Eastings |
| PQ | +0.19 | 108.89 | +0.19 | 55.59 | P | 0.00N | 0.00E |
| QR | +0.24 | -137.46 | +0.24 | 73.44 | Q | 108.89N | 55.59E |
| RS | +0.10 | -61.70 | +0.10 | -20.00 | R | -28.57N | 129.03E |
| ST | +0.09 | -20.41 | +0.09 | -56.31 | S | -90.27N | 109.03E |
| TP | +0.18 | 110.68 | +0.18 | -52.72 | T | -110.68N | 52.72E |
|  | +0.80 |  | +0.80 |  |  | 0.00N | 0.00E |

6.	A traverse survey was carried out along the boundaries of a building plot and 	produced the results as shown in the following table.

| Line | Whole circle bearing | Length (m) |
| --- | --- | --- |
| PQ | 6445 | 121.25 |
| QR | 14326 | 236.83 |
| RS | 23415 | 179.00 |
| SP | 33632 | 264.97 |

Calculate:

- Closing error.

- Coordinates of Q, R and S with respect to P.

| Line | Length (m) | # WCB | RB | Latitude |  | Departure |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Northing | Southing | Easting | Westing |
| PQ | 121.25 | 64°45 | 64°45NE | 51.72 | - | 109.67 | - |
| QR | 236.83 | 143°26 | 36°34SE | - | 190.21 | 141.09 | - |
| RS | 179.00 | 234°15 | 54°15SW | - | 104.58 | - | 145.27 |
| SP | 264.97 | 336°32 | 23°28NW | 243.05 | - | - | 105.52 |
|  | 802.05 |  |  | 294.77 | -294.79 | 250.76 | -250.79 |
|  |  |  | # Error | L= -0.02 |  | D= -0.03 |  |

a)	Closing error 	= [(D) 2 + (L) 2] ½

= 0.0361 m

b)	Correction to latitude and Departure by Bowditch Rule

| Line | Correction to latitude | Corrected latitude | Correction to departure | Corrected departure | Station | Coordinates |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Northings | Eastings |
| PQ | 0.00 | 51.72 | 0.00 | 109.67 | P | 0.00N | 0.00E |
| QR | +0.01 | -190.20 | +0.01 | 141.10 | Q | 51.72N | 109.67E |
| RS | 0.00 | -104.58 | +0.01 | -145.26 | R | -138.48N | 250.77E |
| SP | +0.01 | 243.06 | +0.01 | -105.51 | S | -243.06N | 105.51E |
|  | +0.02 |  | +0.03 |  |  | 0.00N | 0.00E |

7.	The following data refer to an open theodolite traverse (shown in Figure 7) round 	a building site.

| Line | Plan length (m) | Exterior angle | Observed value |
| --- | --- | --- | --- |
| AX | 160.00 |  |  |
| AB | 186.40 | XAB | 33000 |
| BC | 234.00 | ABC | 25130 |
| CD | 170.60 | BCD | 19830 |
| DE | 138.00 | CDE | 28045 |

Station A is the origin. Line AB is pointing north.

Calculate:

- Total coordinates of each station.

- Length and bearing of a proposed drain which is to be laid between the points X and E.

- Bearings of EX and XE to establish the line of the sewer XE by theodolite.

Figure 7

a)

| Line | Length | # Interior Angle | # WCB | RB | Latitude |  | Departure |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | N | S | E | W |
| XA | 160.00 |  | 30°00 | 30°00SW | - | 138.56 | - | 80.00 |
| AB | 186.40 | 30°00 | 00°00 | 0°00N | 186.40 | - | - | - |
| BC | 234.00 | 108°30 | 71°30 | 71°30NE | 74.25 | - | 221.91 | - |
| CD | 170.60 | 161°30 | 90°00 | 90°00E | - | - | 170.60 | - |
| DE | 138.00 | 79°15 | 190°45 | 10°45SW | - | 135.58 | - | 25.74 |

| Line | Latitude | Departure | Station | Coordinates |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Northings | Eastings |
| AB | 186.40 | 0.00 | A | 0.00N | 0.00E |
| BC | 74.25 | 221.91 | B | 186.40N | 0.00E |
| CD | 0.00 | 170.60 | C | 260.65N | 221.91E |
| DE | -135.58 | -25.74 | D | 260.65N | 392.51E |
| EX | 12.49 | 286.77 | E | 125.07N | 366.77E |
| XA | -138.56 | -80.00 | X | 138.56N | 80.00E |

b)

X

y

E

x

y	=	138.56 – 125.07	= 13.49 m

x	=	366.77 – 80.00 	= 286.77 m

	XE	=	(x2 + y2) ½

=	287.09 m

c)

WCB reading of EX	= 270° + tan-1 (y/x)

= 272°4136

WCB reading of XE	= 180° - tan-1 (x/y)

= 92°4136

RB for EX	= 90° - tan-1 (y/x)

= 87°1824NW

RB for XE 	= tan-1 (x/y)

=  87°1824SE

8.	The following staff readings were taken in sequence during a leveling operation 	along a route of a drain.

| Staff reading (m) | Remarks |
| --- | --- |
| 1.250 | BM 1 RL = 60.435 m |
| 1.285 | Peg A |
| 1.125 | Peg B |
| 0.810 | Change point |
| 1.555 |  |
| 1.400 | Inverted staff reading taken to the underside of a bridge |
| 1.235 | Peg C |
| 0.665 | Change point |
| 1.905 |  |
| 0.070 | BM 2 RL = 64.600 m |

Calculate the reduced levels of various points using a standard format.

## Rise/Fall Method

| BS | IS | FS | Rise (+) | Fall (-) | RL | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1.250 | - | - | - | - | 60.435 | ## BM1 |
| - | 1.285 | - | - | 0.035 | 60.400 | Peg A |
| - | 1.125 | - | 0.160 | - | 60.560 | Peg B |
| 1.555 | - | 0.810 | 0.315 | - | 60.875 | Change Pt |
| - | 1.400(I) | - | 2.955 | - | 63.830 | Inverted reading |
| - | 1.235 | - | - | 2.635 | 61.195 | Peg C |
| 1.905 | - | 0.665 | 0.570 | - | 61.765 | Change Pt |
| - | - | 0.070 | 1.835 | - | 63.600 | BM2 |
| 4.710 |  | 1.545 | 5.835 | 2.670 |  |  |

Arithmetic Checks:

 BS -  FS		= 4.710 – 1.545	= 3.165

 Rise -  Fall		= 5.835 – 2.670	= 3.165

Last RL – First RL	= 63.600 – 60.435	= 3.165

	 BS -  FS	=  Rise -  Fall	= Last RL – First RL

Line of Collimation Method

| BS | IS | FS | Height of Collimation | RL | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1.250 | - | - | 61.685 | 60.435 | BM1 |
| - | 1.285 | - |  | 60.400 | Peg A |
| - | 1.125 | - |  | 60.560 | Peg B |
| 1.555 | - | 0.810 | 62.430 | 60.875 | Change Pt |
| - | 1.400(I) | - |  | 63.830 | Inverted reading |
| - | 1.235 | - |  | 61.195 | Peg C |
| 1.905 | - | 0.665 | 63.670 | 61.765 | Change Pt |
| - | - | 0.070 |  | 63.600 | BM2 |
| 4.710 |  | 1.545 |  |  |  |

Arithmetic Checks:

 BS -  FS		= 4.710 – 1.545	= 3.165

Last RL – First RL	= 63.600 – 60.435	= 3.165

	 BS -  FS	= Last RL – First RL

9.	The table below shows the results of a leveling operation along the centre line of a 	roadway where settlement has taken place.  The road was initially constructed at a 	uniform gradient rising at 1 in 75 from A to B. Assuming no settlement has taken 	place at station A.

| # BS | IS | FS | Remarks | Distance (m) |
| --- | --- | --- | --- | --- |
| 3.540 |  |  | BM 1 RL = 78.675 m |  |
| 0.410 |  | 3.665 |  |  |
| 0.525 |  | 2.245 |  |  |
|  | 2.840 |  | Station A | 0 |
|  | 2.440 |  |  | 30 |
|  | 2.045 |  |  | 60 |
| 2.475 |  | 1.655 |  | 90 |
|  | 2.090 |  |  | 120 |
|  | 1.700 |  |  | 150 |
|  | 1.315 |  |  | 180 |
|  | 0.900 |  |  | 210 |
| 1.465 |  | 0.485 |  | 240 |
|  | 2.055 |  |  | 270 |
|  | 1.645 |  | Station B | 300 |
|  |  | 2.040 | BM 2 RL = 78.000 m |  |

- Compute the reduced levels of various points using a standard format.

- Compute the maximum settlements along the line AB.

- Compute the difference in elevation between A and B.

a)	Rise/Fall Method

| BS | IS | FS | Rise (+) | Fall (-) | RL | Distance | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.540 | - | - | - | - | 78.675 | - | # BM1 |
| 0.410 | - | 3.665 | - | 0.125 | 78.550 | - |  |
| 0.525 | - | 2.245 | - | 1.835 | 76.715 | - |  |
| - | 2.840 | - | - | 2.315 | 74.400 | 0 | Station A |
| - | 2.440 | - | 0.400 | - | 74.800 | 30 |  |
| - | 2.045 | - | 0.395 | - | 75.195 | 60 |  |
| 2.475 | - | 1.655 | 0.390 | - | 75.585 | 90 |  |
| - | 2.090 | - | 0.385 | - | 75.970 | 120 |  |
| - | 1.700 | - | 0.390 | - | 76.360 | 150 |  |
| - | 1.315 | - | 0.385 | - | 76.745 | 180 |  |
| - | 0.900 | - | 0.415 | - | 77.160 | 210 |  |
| 1.465 | - | 0.485 | 0.415 | - | 77.575 | 240 |  |
| - | 2.055 | - | - | 0.590 | 76.985 | 270 |  |
| - | 1.645 | - | 0.410 | - | 77.395 | 300 | Station B |
| - | - | 2.040 | - | 0.395 | 77.000 | - | BM2 |
| 8.415 |  | 10.090 | 3.585 | 5.260 |  |  |  |

Arithmetic Checks:

 BS -  FS		= 8.415 – 10.090	= -1.675

 Rise -  Fall		= 3.585 – 5.260	= -1.675

Last RL – First RL	= 77.000 – 78.675	= -1.675

	 BS -  FS	=  Rise -  Fall	= Last RL – First RL

Line of Collimation Method

| BS | IS | FS | Height of Collimation | RL | Distance | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 3.540 | - | - | 82.215 | 78.675 | - | BM1 |
| 0.410 | - | 3.665 | 78.960 | 78.550 | - |  |
| 0.525 | - | 2.245 | 77.240 | 76.715 | - |  |
| - | 2.840 | - |  | 74.400 | 0 | Station A |
| - | 2.440 | - |  | 74.800 | 30 |  |
| - | 2.045 | - |  | 75.195 | 60 |  |
| 2.475 | - | 1.655 | 78.060 | 75.585 | 90 |  |
| - | 2.090 | - |  | 75.970 | 120 |  |
| - | 1.700 | - |  | 76.360 | 150 |  |
| - | 1.315 | - |  | 76.745 | 180 |  |
| - | 0.900 | - |  | 77.160 | 210 |  |
| 1.465 | - | 0.485 | 79.040 | 77.575 | 240 |  |
| - | 2.055 | - |  | 76.985 | 270 |  |
| - | 1.645 | - |  | 77.395 | 300 | Station B |
| - | - | 2.040 |  | 77.000 | - | BM 2 |
| 8.415 |  | 10.090 |  |  |  |  |

Arithmetic Checks:

 BS -  FS		= 8.415 – 10.090	= -1.675

Last RL – First RL	= 77.000 – 78.675	= -1.675

	 BS -  FS	= Last RL – First RL

- Vertical drop between 30 m between A and B = 30 / 75 = 0.4 m

| # Remark | Top level of the given gradient | RL | Settlement | Distance |
| --- | --- | --- | --- | --- |
| # Station A | 74.400 | 74.400 | 0.000 | 0 |
|  | 74.800 | 74.800 | 0.000 | 30 |
|  | 75.200 | 75.195 | 0.005 | 60 |
|  | 75.600 | 75.585 | 0.015 | 90 |
|  | 76.000 | 75.970 | 0.030 | 120 |
|  | 76.400 | 76.360 | 0.040 | 150 |
|  | 76.800 | 76.745 | 0.055 | 180 |
|  | 77.200 | 77.160 | 0.040 | 210 |
|  | 77.600 | 77.575 | 0.025 | 240 |
|  | 78.000 | 76.985 | 1.015 | 270 |
| Station B | 78.400 | 77.395 | 1.005 | 300 |

Max settlement = 1.015 m

c)	Reduced level at A		=	74.400 m

Reduced level at B		=	77.395 m

Elevation between A and B	=	77.395 – 74.400

=	2.995 m

10.	The following table shows the staff readings taken to determine the clearance 		between the river level and the soffit of a road bridge. Reduce the levels and 	determine the clearance between the river level and the soffit of the bridge.

| # BS | IS | FS | RL | Remarks |
| --- | --- | --- | --- | --- |
| 0.872 |  |  | 21.460 | ## BM |
| 0.665 |  | 3.980 |  |  |
|  | 2.920 |  |  | River level at A |
|  | -1.332 |  |  | Soffit of bridge at A |
|  | -1.312 |  |  | Soffit of bridge at B |
|  | -1.294 |  |  | Soffit of bridge at C |
|  | -1.280 |  |  | Soffit of bridge at D |
|  | 2.920 |  |  | River level at D |
| 4.216 |  | 0.597 |  |  |
|  |  | 1.155 |  | Permanent object |

| # BS | IS | FS | RL | Remarks |
| --- | --- | --- | --- | --- |
| 0.872 |  |  | 21.460 | ## BM |
| 0.665 |  | 3.980 | 18.352 |  |
|  | 2.920 |  | 16.097 | River level at A |
|  | -1.332 |  | 20.349 | Soffit of bridge at A |
|  | -1.312 |  | 20.329 | Soffit of bridge at B |
|  | -1.294 |  | 20.311 | Soffit of bridge at C |
|  | -1.280 |  | 20.297 | Soffit of bridge at D |
|  | 2.920 |  | 16.097 | River level at D |
| 4.216 |  | 0.597 | 18.420 |  |
|  |  | 1.155 | 21.481 | Permanent object |
| 5.573 |  | 5.732 |  |  |

Arithmetic Checks:

 BS -  FS		= 5.573 – 5.732	= 0.021

Last RL – First RL	= 21.481 – 21.460	= 0.021

	 BS -  FS	= Last RL – First RL

- Table below shows a page of a level book in which certain entries are missing.  Complete the missing entries and carry out the normal checks.

| BS | IS | FS | Rise | Fall | Reduced Levels |
| --- | --- | --- | --- | --- | --- |
| 3.786 |  |  |  |  | 36.642 |
|  | - |  | 2.474 |  | - |
|  | 1.960 |  |  | 0.648 | 38.468 |
| - |  | 3.560 |  | - | 36.868 |
|  | 3.698 |  |  | - | 34.042 |
|  | 0.670 |  | - |  | - |
| - |  | 2.180 |  | - | 35.560 |
|  | 1.052 |  | 1.186 |  | - |
| 2.874 |  | - |  | - | 34.992 |
|  | - |  | 1.158 |  | - |
|  | 0.950 |  | 0.766 |  | 36.916 |
|  |  | 1.412 |  | - | - |

| BS | IS | FS | Rise | Fall | Reduced Levels |
| --- | --- | --- | --- | --- | --- |
| 3.786 |  |  |  |  | 36.642 |
|  | 1.312 |  | 2.474 |  | 39.116 |
|  | 1.960 |  |  | 0.648 | 38.468 |
| 0.872 |  | 3.560 |  | 1.600 | 36.868 |
|  | 3.698 |  |  | 2.826 | 34.042 |
|  | 0.670 |  | 3.028 |  | 37.070 |
| 2.238 |  | 2.180 |  | 1.510 | 35.560 |
|  | 1.052 |  | 1.186 |  | 36.746 |
| 2.874 |  | 2.806 |  | 1.754 | 34.992 |
|  | 1.716 |  | 1.158 |  | 36.150 |
|  | 0.950 |  | 0.766 |  | 36.916 |
|  |  | 1.412 |  | 0.462 | 36.454 |
| 9.770 |  | 9.958 | 8.612 | 8.800 |  |

Arithmetic Checks:

 BS -  FS		= 9.770 – 9.958	= -0.188

 Rise -  Fall		= 8.612 – 8.800	= -0.188

Last RL – First RL	= 36.454 – 36.642	= -0.188

	 BS -  FS	=  Rise -  Fall	= Last RL – First RL

- The following table shows the staff readings obtained from a survey along the line of a proposed roadway from A to B.  Reduce the levels from A to B applying all the appropriate checks.  Given that the finished roadway has to be evenly graded from A to B, calculate the depths of cut or fill at 20m intervals between A and B.

| BS | IS | FS | HPC or |  | Surface Reduced Level | Grade Reduced Level | Fill | Cut | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rise | Fall |  |  |  |  |  |
| 0.824 |  |  |  |  | 39.220 |  |  |  | TBM No.1 |
|  | 1.628 |  |  |  |  |  |  |  | A |
|  | 0.790 |  |  |  |  |  |  |  | 20m from A |
|  | 0.383 |  |  |  |  |  |  |  | 40m from A |
| 2.154 |  | 1.224 |  |  |  |  |  |  | 60m from A |
|  | 2.336 |  |  |  |  |  |  |  | 80m from A |
|  | 2.757 |  |  |  |  |  |  |  | 100m from A |
| 2.555 |  | 0.461 |  |  |  |  |  |  | Change Point |
|  | 2.275 |  |  |  |  |  |  |  | 120m from A |
|  | 0.436 |  |  |  |  |  |  |  | 140m from A |
|  | 0.227 |  |  |  |  |  |  |  | 160m from A |
|  | 0.716 |  |  |  |  |  |  |  | 180m from A |
|  | 0.652 |  |  |  |  |  |  |  | B. 200m from A |
|  |  | 0.233 |  |  |  |  |  |  | TBM No.2 |

Rise/Fall Method

| BS | IS | FS | Rise (+) | Fall (-) | Surface Reduced Level | Grade Reduced Level | Fill | Cut | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0.824 |  |  |  |  | 39.220 |  |  |  | TBM No.1 |
|  | 1.628 |  |  | 0.804 | 38.416 | 38.416 |  |  | A |
|  | 0.790 |  | 0.838 |  | 39.254 | 38.816 |  | 0.438 | 20m from A |
|  | 0.383 |  | 0.407 |  | 39.661 | 39.216 |  | 0.445 | 40m from A |
| 2.154 |  | 1.224 |  | 0.841 | 38.820 | 39.616 | 0.796 |  | 60m from A |
|  | 2.336 |  |  | 0.182 | 38.638 | 40.016 | 1.378 |  | 80m from A |
|  | 2.757 |  |  | 0.421 | 38.217 | 40.416 | 2.199 |  | 100m from A |
| 2.555 |  | 0.461 | 2.296 |  | 40.513 |  |  |  | Change Point |
|  | 2.275 |  | 0.280 |  | 40.793 | 40.816 | 0.023 |  | 120m from A |
|  | 0.436 |  | 1.839 |  | 42.632 | 41.216 |  | 1.416 | 140m from A |
|  | 0.227 |  | 0.209 |  | 42.841 | 41.616 |  | 1.225 | 160m from A |
|  | 0.716 |  |  | 0.489 | 42.352 | 42.016 |  | 0.336 | 180m from A |
|  | 0.652 |  | 0.064 |  | 42.416 | 42.416 |  |  | B. 200m from A |
|  |  | 0.233 | 0.419 |  | 42.835 |  |  |  | TBM No.2 |
| 5.533 |  | 1.918 | 6.532 | 2.737 |  |  |  |  |  |

Arithmetic Checks:

 BS -  FS		= 5.533 – 1.918	= 3.615

 Rise -  Fall		= 6.352 – 2.737	= 3.615

Last RL – First RL	= 42.835 – 39.220	= 3.615

	 BS -  FS	=  Rise -  Fall	= Last RL – First RL

To calculate depths of cuts or fills,

Gradient of roadway 	= (42.416 – 38.416) / 200 = 1/50

Rise over any length	= 1/50 of length

	Rise over 20 m length	= 1/50  20 m = 0.400 m

13.	Table below shows levels taken along the centre line of a proposed sewer.  The 	sewer at 0 m has an invert level of 88.900 m and is to fall towards distance 50 m 	at 1 in 100.

a.	Calculate the reduced levels of different points using a standard format 	and apply appropriate checks.

b.	Draw a vertical section along the centre line of the sewer on a horizontal 	scale of 1:500 and vertical of 1:50.

c.	From the section, compute the depth of cover at each distance point.

| BS | IS | FS | Rise | Fall | Reduced level | Distance | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.670 |  |  |  |  | 92.550 |  | BM |
| 1.520 |  | 3.870 |  |  |  | 0 | Ground level |
|  | 0.910 |  |  |  |  | 10 | Ground level |
|  | 1.590 |  |  |  |  | 20 | Ground level |
|  | 1.770 |  |  |  |  | 30 | Ground level |
|  | 1.660 |  |  |  |  | 40 | Ground level |
|  | -4.200 |  |  |  |  | 50 | Underside of bridge |
|  |  | 0.720 |  |  |  | 50 | Ground level |

(a)

| BS | IS | FS | Rise | Fall | Reduced level | Distance | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.670 |  |  |  |  | 92.550 |  | BM |
| 1.520 |  | 3.870 |  | 2.200 | 90.350 | 0 | Ground level |
|  | 0.910 |  | 0.610 |  | 90.960 | 10 | Ground level |
|  | 1.590 |  |  | 0.680 | 90.280 | 20 | Ground level |
|  | 1.770 |  |  | 0.180 | 90.100 | 30 | Ground level |
|  | 1.660 |  | 0.110 |  | 90.210 | 40 | Ground level |
|  | -4.200 |  | 5.860 |  | 96.070 | 50 | Underside of bridge |
|  |  | 0.720 |  | 4.920 | 91.150 | 50 | Ground level |
| 3.190 |  | 4.590 | 6.580 | 7.980 |  |  |  |

Arithmetic Checks:

 BS -  FS		= 3.190 – 4.590	= -1.400

 Rise -  Fall		= 6.580 – 7.980	= -1.400

Last RL – First RL	= 91.150 – 92.550	= -1.400

	 BS -  FS	=  Rise -  Fall	= Last RL – First RL

(b)	Gradient of sewer = 1/100

Vertical drop over 10 m = 10/100 = 0.1 m

Diameter of sewer = 1 m

Thickness of sewer = 25 mm

Reduced level of centre line of sewer = Invert level + 0.5

| Distance | Invert Level | # Reduced Level of Centre Line |
| --- | --- | --- |
| 0 | 88.900 | 89.400 |
| 10 | 88.800 | 89.300 |
| 20 | 88.700 | 89.200 |
| 30 | 88.600 | 89.100 |
| 40 | 88.500 | 89.000 |
| 50 | 88.400 | 88.900 |

c)	Reduced level of top of sewer = Reduced level of centre line + 0.5 + 0.025

Depth of cover = Reduced level of ground – Reduced level of top of sewer

| Distance | Reduced Level of Ground | Reduced Level of Top of Sewer | Depth of Cover (m) |
| --- | --- | --- | --- |
| 0 | 92.350 | 89.925 | 0.425 |
| 10 | 90.960 | 89.825 | 1.135 |
| 20 | 90.280 | 89.725 | 0.555 |
| 30 | 90.100 | 89.625 | 0.475 |
| 40 | 90.210 | 89.525 | 0.685 |
| 50 | 91.150 | 89.425 | 1.725 |

14.	Table below shows the results of a leveling operation for the construction of a 	new road.  Complete the table and apply all necessary checks.

The roadway is to be regraded from A to C; both of whose present levels are to remain unaltered.

a.	Calculate the gradient of AC and the depth of cut or fill required at B.

b.	Choose any suitable scales and draw a vertical section from A to C.

c.	Draw cross-sections at the points of maximum cut and maximum fill to show the road construction given that the road is 5 m wide with 45 side slopes.

| BS | IS | FS | Rise | Fall | Reduced level | Distance | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.500 |  |  |  |  |  | 0 m | At A |
|  | 2.340 |  |  |  |  | 15 |  |
|  | 2.640 |  |  |  |  | 30 |  |
|  | 2.130 |  |  |  |  | 45 |  |
| 0.830 |  | 2.580 |  |  |  | 60 |  |
|  | 0.930 |  |  |  | 57.750 | 62.5 | At B |
|  | 0.340 |  |  |  |  | 75 |  |
|  | 1.790 |  |  |  |  | 90 |  |
| 1.300 |  | 2.530 |  |  |  | 105 |  |
|  | 0.660 |  |  |  |  | 120 |  |
|  | 1.920 |  |  |  |  | 135 |  |
|  |  | 1.100 |  |  |  | 150 | At C |

| BS | IS | FS | Rise | Fall | Reduced level | Distance | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.500 |  |  |  |  | 57.930 | 0 m | At A |
|  | 2.340 |  | 0.160 |  | 58.090 | 15 |  |
|  | 2.640 |  |  | 0.300 | 57.790 | 30 |  |
|  | 2.130 |  | 0.510 |  | 58.300 | 45 |  |
| 0.830 |  | 2.580 |  | 0.450 | 57.850 | 60 |  |
|  | 0.930 |  |  | 0.100 | 57.750 | 62.5 | At B |
|  | 0.340 |  | 0.590 |  | 58.340 | 75 |  |
|  | 1.790 |  |  | 1.450 | 56.890 | 90 |  |
| 1.300 |  | 2.530 |  | 0.740 | 56.150 | 105 |  |
|  | 0.660 |  | 0.640 |  | 56.790 | 120 |  |
|  | 1.920 |  |  | 1.260 | 55.530 | 135 |  |
|  |  | 1.100 | 0.820 |  | 56.350 | 150 | At C |
| 4.630 |  | 6.210 | 2.720 | 4.300 |  |  |  |

Arithmetic Checks:

 BS -  FS		= 4.630 – 6.210	= -1.580

 Rise -  Fall		= 2.720 – 4.300	= -1.580

Last RL – First RL	= 56.350 – 57.930	= -1.580

	 BS -  FS	=  Rise -  Fall	= Last RL – First RL

(a)	Gradient of AC	=	(Vertical Distance) / (Horizontal Distance)

=	(57.930 – 56.350) / 150

=	1.580 / 150

=	1.053  10-2

A

x / 1.580 = 62.5 / 150

x

1.580

x = 0.658

B

Required level at B	=	57.750 – 0.658

62.5

=	57.272 m

C

RL at B		=	57.750 m

Depth of cut at B	=	57.750 – 57.272

150

=	0.478 m

(b)

(c)

15.	The following tacheometric observations were made along the centre of a 	proposed roadway at 20m intervals.

Instrument station D (at 60m distance)

Instrument height = 1.45m

Reduced level of station D = 234.21m.

| Target station | Vertical angle | Stadia readings |  |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  | Top | Mid | Bottom |  |
| A | -420 | 1.750 | 1.450 | 1.140 | Distance 0 m |
| B | -420 | 1.400 | 1.200 | 1.000 | Distance 20 m |
| C | -420 | 1.210 | 1.105 | 1.000 | Distance 40 m |
| E | -300 | 1.200 | 1.100 | 1.000 | Distance 80 m |
| F | -300 | 1.650 | 1.450 | 1.250 | Distance 100 m |

Instrumental constants m = 100, k = 0.

The new roadway is to begin at point A and rise at 1 in 50 towards B.  Calculate for each station:

- Reduced level

- Proposed roadway level

- Depth of cutting required.

a)

| Station | Reduced level (m) |
| --- | --- |
| A | 229.614 |
| B | 231.446 |
| C | 232.973 |
| D | 234.210 |
| E | 233.515 |
| F | 232.119 |

Sample calculation for station A:

HI	= 1.45 m

hA	= 1.450 m

SA  	= 1.750 - 1.140 = 0.610 m

VA 	= 50(SA)(sin 2)

= 50 (0.610) [sin 2(-420)]

= -4.596 m

	RL of A 	= 234.21 - VA - hA + HI

= 234.21 – 4.596 – 1.450 +1.45

= 229.614 m

- Gradient of AF		= 1/50

Vertical rise over 20m		= 20/50	= 0.4 m

| Target Station | Proposed Level (m) |
| --- | --- |
| A | 229.614 |
| B | 230.014 |
| C | 230.414 |
| D | 230.814 |
| E | 231.214 |
| F | 231.614 |

c)

| # Target Station | Reduced Level (m) | Proposed Level (m) | Depth of Cut (m) |
| --- | --- | --- | --- |
| A | 229.614 | 229.614 | 0 |
| B | 231.446 | 230.014 | 1.432 |
| C | 232.973 | 230.414 | 2.559 |
| D | 234.210 | 230.814 | 3.396 |
| E | 233.515 | 231.214 | 2.301 |
| F | 232.119 | 231.614 | 0.505 |

16.	The following table shows the tacheometric observations using a theodolite with 	zero additive constant and a multiplying constant of 100.

Calculate the horizontal length AB and reduced level of point B.

| Station point | Reduced level of station | Height of Instrument | Staff point | Vertical angle | Stadia readings | Horizontal length | Reduced level of point | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 100.0 | 1.49 | B | +55142 | 1.076 |  |  | Staff vertical |
|  |  |  |  |  | 1.000 |  |  |  |
|  |  |  |  |  | 0.923 |  |  |  |
| A | 100.0 | 1.49 | B | +55142 | 1.000 |  |  | Mid hair readings |
|  |  |  |  | +93406 | 2.000 |  |  | Staff vertical |

## Intercept, S	=	1.076 - 0.923

## = 	0.153 m

	Horizontal length AB	= 100Scos2

= 100(0.153) cos2 (55142)

= 15.140 m

V	= 50Ssin2

= 50(0.153) sin 2(55142)

= 1.554 m

	Reduced level of B 	= 100.0 + HI + V – h

= 100.0 + 1.49 + 1.554 – 1.000

= 102.044 m

Alternatively,

Horizontal length AB	=	          S   		=	                   1.000

tan 2 – tan 1 		 tan 93406 - tan 55142

=	15.713 m

Required level at B	=	Required level at A + L

=	100.0 + HI + V – h

=	100.0 + 1.49 + (15.173) tan (55142) –1.000

=	102.048 m

17.	The readings were taken along the route of a sewer.

| BS | IS | FS | Ht. of coll. | RL | Distance (m) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 0.860 |  |  |  | 97.240 |  | TBM |
| 2.240 |  |  |  |  | 0 | IP 5 |
|  | 2.590 |  |  |  | 48.000 | IP 4 |
|  | 1.690 |  |  |  | 64.000 | IP 3 |
| 1.322 |  | 0.975 |  |  | 85.600 | IP 2 |
|  |  | 0.937 |  |  | 118.400 | IP 1 |

IP = Inspection Pit

- Calculate the reduced levels of IP 5 to IP 1.

- Invert level IP 1 is to be 96.8111m and the sewer runs are to slope down at the following gradients:

IP 1 to IP 2	1 in 125

IP 2 to IP 3	1 in 115

IP 3 to IP 4	1 in 100

IP 4 to IP 5	1 in 80

Calculate the invert levels of each inspection pit.

a)

| BS | IS | FS | Ht. of coll. | # RL | Distance (m) | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 0.860 |  |  | 98.100 | 97.240 |  | TBM |
| 2.240 |  | 0.550 | 99.790 | 97.550 | 0 | IP5 |
|  | 2.590 |  |  | 97.200 | 48.000 | IP4 |
|  | 1.690 |  |  | 98.100 | 64.000 | IP3 |
| 1.322 |  | 0.975 | 100.137 | 98.815 | 85.600 | IP2 |
|  |  | 0.937 |  | 99.200 | 118.400 | IP1 |
| 4.422 |  | 2.462 |  |  |  |  |

Arithmetic Checks:

 BS -  FS		= 4.422 – 2.462	= 1.960

Last RL – First RL	= 99.200 – 97.240	= 1.960

	 BS -  FS	=  Rise -  Fall	= Last RL – First RL

b)

| Station | Invert Level (m) |
| --- | --- |
| IP1 | 96.811 |
| IP2 | 96.549 |
| IP3 | 96.361 |
| IP4 | 96.201 |
| IP5 | 95.601 |

Sample Calculation for Station IP2:

Distance between IP1 and IP2	= 118.400 – 85.600

= 32.800 m

Vertical drop over IP1 and IP2	= (32.800)/125

= 0.2624 m

Invert level of IP2	= 96.811 - 0.2624

= 96.549 m

18.	The following vertical angles were measured in a tacheometric survey of a plot of 	land.

| Instrument station | Target station | Vertical angle |
| --- | --- | --- |
| A | E | +0420 |
| B | D | -0230 |
| C | C | +0515 |
| C | B | -0425 |
| B | A | -0626 |
| A | C | +0728 |
| E | C | +0331 |

Given that the reduced level of station A is 156.20 m, and that the height of the target was adjusted to be the same as the theodolite for each sight, calculate the reduced level of the stations B, C, D and E.

Assuming all distance given is the horizontal distance.

## E

278.67

## A

Vertical distance from pt A to pt E	= 278.67  tan 420

= 21.116 m

	RL of pt E = 156.20 + 21.116	= 177.32 m

177.32

## E

## D

Vertical distance from pt E to pt D 	= 226.34  tan (-230)

= -9.882 m

	RL of pt D = 177.316 - 9.882 = 167.43m

##### C

## D

278.07

## Vertical distance from pt D to pt C	= 278.07  tan 515

= 25.551 m

	RL of pt C = 167.433 + 25.551 = 192.98 m

256.88

## C

## B

Vertical distance from pt C to pt B	= 256.88  tan (-425)

= -19.841 m

	RL of pt B = 192.984 – 19.841 = 173.14 m

## A

## B

150.20

Vertical distance from pt B to pt A 	= 150.20  tan 626

= 16.936 m

- RL of pt A = 173.143 + 16.936 = 156.20 m

19.	Figure 19 shows 2 theodolite stations A and B set up some distance from a 	vertical wall. All relevant survey observations for finding the height of the wall 	are shown in the figure. Calculate the height of the wall CD.

## C

B’

4450

## D

## B

5

16

E

49.0 m

Figure 19

3010

## A

BCD	= 180 - 90 - 4450	= 4510

ACD	= 180 - 3010	= 5950

ACB	= ACD - BCD	= 1440

BAC	= 3010 - 16		= 1410

By Sine Rule,

AB / sinACB	= BC / sinBAC

49.0 / sin 1440	= BC / sin 1410

 BC			= 47.364 m

CE	= BC sin 4450	= 33.394 m

BE	= BC cos 4450	= 33.589 m

DE	= BE tan 5		= 2.939 m

	CD	= CE – DE	= 30.46 m

20.	While conducting tacheometric survey of 3 lines AB, BC and CD, the vertical 	angles were measured reciprocally as follows:

| Line | Vertical angle | Length (m) |
| --- | --- | --- |
| AB | +033520 | 87.35 |
| BA | -033600 |  |
| BC | +011000 | 153.25 |
| CB | -011040 |  |
| CD | +053520 | 93.57 |
| DC | -053600 |  |

Calculate the reduced levels of stations B, C and D. Given the reduced level of A = 135.70 m.

Assume:  Target height = Instrument heights on all sightings.

| Station | Line | Vertical angle | RL (m) | Average RL |
| --- | --- | --- | --- | --- |
| B | AB | +033520 | 141.179 | 141.19 |
|  | BA | -033600 | 141.196 |  |
| C | BC | +011000 | 144.300 | 144.32 |
|  | CB | -011040 | 144.347 |  |
| D | CD | +053520 | 153.456 | 153.49 |
|  | DC | -053600 | 153.522 |  |

Sample calculation for Station B:

RL of B (using line AB)	= RL of A + 87.36 tan 33520

= 141.179 m

RL of B (using line BA) 	= RL of A + 87.36 tan 33600

= 141.196 m

Average RL of B		= (141.179 + 141.196)  2

= 141.188 m

###### The End