# 1.	Abstract

Soil settles because of consolidation, which is the gradual reduction in volume of a fully saturated soil due to drainage of pore water. Soil mechanics studies the consolidation of soils to help estimate the degree of ground movement when a structure is built on it so that the stability of the structure can be known.

Permeability of a soil is its ability to allow seepage of fluids through its interconnected void spaces. The permeability of the soil also affects the choice and design of foundations.

In the experiment, we have investigated the characteristics of Singapore Marine Clay using the one-dimensional consolidation test.  The Virgin Curve and Unloading Curve of the sample was plotted and we found that the gradient of the Virgin Curve is 0.417 and it is lower than theoretical values.  Reasons for the difference will be discussed.  In the test, the water content of the sample varied from 53.73% to 53.47% while the void ratio decreased from 1.289 to 1.077.  From the relevant graphs plotted, an estimation of the pre-consolidation pressure is 71.5 kPa.  The shape of the graph indicates that once the soil is subject to a vertical loading pressure and is consolidated, removing the load will not allow the soil to go back to its original state as the soil is permanently consolidated.

In the permeability test, the permeability of coarse sand and marine clay were compared using two different methods - the Constant Head Test and the Falling Head Test.  From the experiment, it is clear that coarse sand has a much higher permeability (about 1 million times) than clay.

# 2.	Acknowledgement

Sincere thanks to Professor Karu who provided us with detailed explanations of the experiment.  We also appreciate the guidance of Mr. Huang Zee Meng during the experiment, not forgetting Madam Jamilah who compiled and mailed us the experimental results.

# 3.	Objectives

- To determine the consolidation characteristics of Singapore Marine Clay using a one-dimensional consolidation test.

- To obtain the permeability of soils using Falling Head Permeability Test and Constant Head Permeability Test.

# 4.	Experimental Results & Discussion

4.1	Assumptions.

- The clay layer is homogenous.

- The clay layer is saturated.

- The coefficient of consolidation is constant during the consolidation process.

- The compression of the soil layer is due to the squeezing of water out of the void spaces.  Particle sizes remain the same.

- Deformation of soil occurs only in the direction of the load application.

- Deformation of apparatus (permeable sandstone) is negligible.

- Assume specimen is under uniform stress, and applied instantaneously.  (In actual application, the soil is under uniform stress across 20 km for a depth of 10 m.)

- Permeability of filter paper used in the consolidation apparatus is higher than that of the soil sample, and that no soil particles escape to the porous plates.

4.2	Summary of Results Obtained.

4.2.1	Experimental Values of Compression Index, Cc & Swelling Index, CS.

| Compression index, Cc | 0.417 |
| --- | --- |
| Swelling index, Cs | 0.160 |
| Pre-consolidation Pressure, Pc’ | 71.5 kPa |

    - Comparison between Void Ratio vs. Log v’ and Volumetric Strain vs. Log v’.

From the two graphs, it is clear that the Volumetric Strain Vs Log v’ is the exact reflection of the Void Ratio vs. Log v’ graph.  Volumetric strain is inversely proportional to void ratio.

    - Correlated Values.

| Type of Soil | Formula | Value of Cc | Reference |
| --- | --- | --- | --- |
| Remoulded | 0.007 (LL – 10%) | 0.5915 | Terzaghi & Peck |
| Undisturbed | 0.009 (LL – 10%) | 0.7605 | Terzaghi & Peck |
| Remoulded, based on Critical State Concept | - 0.01346 PI - 0.02116 (PL – 9%) - 0.00828 (LL – 9%) | 0.7984 0.5540 0.7079 | Schofield & Wroth |
| Based on Critical State Concept | 0.05 Gs PI | 0.7860 | Wood |

4.2.4	Comparison between experimental values and correlated values of Cc.

The Cc value 0.417 obtained in this experiment is lower than any of the theoretical values as computed above.  This may be due to the fact that the theoretical values were calculated using Liquid and Plastic Limits from previous testing and the sample in this case may have different values.

Also, the theoretical values were derived for undisturbed clay, and since our sample has been removed from the site for a period of time, the values would be different.

There is also a possibility that the sample was taken from a deep portion from the site.  Soil at a deep part of the site is subjected to a high pressure.  Therefore, the void ratio against vertical stress graph we obtained is actually a reloading curve and not a virgin curve.  The ratio Cc that we obtained is actually a value of Cr, and is therefore lower.  To eliminate the possibility of this error, we could have allowed loading to go beyond 200kPa.

4.2.5	Consolidation Parameters.

| Pressure, P (kPa) | Void Ratio, e | mv (m2/MN) | Cv (m2/yr) | kv (cm/s) |
| --- | --- | --- | --- | --- |
| 25 | 1.289 | 0.7518 | 2.621 | 1.114 X 10-7 |
| 50 | 1.247 | 0.7314 | 1.674 | 3.635 X 10-8 |
| 100 | 1.168 | 0.7008 | 0.9544 | 2.415 X 10-8 |
| 200 | 1.018 | 0.6940 | 0.6399 | 1.390 X 10-8 |
| 50 | 1.077 | 0.1943 | - | - |

As the effective stress increases, the permeability of the soil decreases.  As the load increases, there will be a reduction in the void ratio as the soil becomes more compacted.  Time taken for the water to drain out from the soil takes a longer time as permeability decreases.

4.2.6	Permeability Tests.

| Type of Test | Soil Type | Coefficient of Permeability, kz (cm/s) |
| --- | --- | --- |
| Constant Head Test | Coarse Sand | 0.0742 |
| Falling Head Test | Singapore Marine Clay | 5.65 X 10-7 |

The coarse sand has medium degree of permeability while Singapore Marine Clay has a very low degree of permeability.

  - Validity & Reliability of Results.

    - One-Dimensional Consolidation Test.

The results show the trends of coefficient of permeability, coefficient of consolidation and coefficient of volume change as effective stress changes.  However, errors could have caused the variation of these parameters for a given effective stress.  Examples of errors are external vibration on the odeometer and error in estimating the best slope from the graph.

- Discrepancy in values of Cv.

Two values of Cv were obtained through two different methods (the logarithmic curve-fitting method and the square root time curve-fitting method).  By right, we should obtain the same results for both methods.  However, there could be some errors in locating the best slope for the graph.  The value obtained through the logarithmic curve-fitting method would be more prone to errors since values were estimated by human judgement.

- Value of Cs.

Under the BS1377 code, section 3.5.3.1, the number of unloading stages should be at least half the number of loading stages.  Thus the reliability of the swelling curve we obtained is in suspect because we only had time to perform one unloading stage.

- Permeability Constant.

The permeability constant was calculated using the average of the two Cv results obtained.  Hence the inaccuracy of the permeability constant will be lower. The permeability constant obtained sort of coincides with that of clay, so we can say that the results obtained should be reliable.

    - Permeability Tests.

The permeability constant obtained for the coarse sand and Singapore Marine Clay is in the order that looks logical according to the properties of coarse sand and clay that we know.  The permeability constant obtained through the one dimension consolidation test is fairly consistent with the results obtained by the falling head test.  Hence, results obtained should be valid.

4.4	Recommended Design Values of Singapore Marine Clay.

The recommended values are as follows:

| Cc | Cr | mv (m2/MN) | Cv (m2/yr) | kv (10-8 cm/s) |
| --- | --- | --- | --- | --- |
| 0.40 – 0.45 | 0.10 – 0.20 | 0.10 – 0.80 | 0.50 – 3.00 | 1.00 – 12.0 |

The value of Cr is based on the Cs value that is similar because of the similar gradients of the swelling curve and the reloading curve.

All values are based on the experimental results obtained in this experiment.  The range of the values is large to take into consideration the different structural patterns and loading conditions.  This is especially so for the value of Cv because the predicted settlement times can be greatly in excess of those observed in practice.  Also, the small size of the specimen used for the test does not represent adequately the fabric features found in many natural deposits, which collectively dominate the drainage characteristics of the soil en masse and therefore the rate of settlement in-situ.  (BS1377: Part 5: 1990, 3.1.1)

4.5	Estimation of E’.

| P | Volumetric Strain | E' |
| --- | --- | --- |
| 25 | 0.01879 | 1330 |
| 50 | 0.03674 | 1361 |
| 100 | 0.07049 | 1419 |
| 200 | 0.1350 | 1481 |

On average, Young’s modulus of Singapore Marine Clay, E’ = 1450 kPa.

The values of volumetric strains for P = 25kPa and 50kPa are not used because of pre-consolidation stress.  We have estimated the pre-consolidation stress to be 71.5 kPa. This means that the 25kPa and 50kPa values cannot be used as the sample have previously been loaded to that magnitude before, and the E estimated will be larger than the actual value because the soil will be 'stiffer' due to consolidation.

4.6	Other Possible Sources of Error.

The ring may not be uniformly compressed in the soil sample.  Uneven distribution of loading would cause some voids to occur at certain parts.

The soil sample was cut by hand.  Compression might have affected the side, top and bottom of the ring.  A trimming machine would minimize the amount of disturbance caused.  The manual cutting might have caused the top and bottom of the sample to be un-level.  Then, the sample would not be uniformly loaded.

# 5.	Conclusion

In the above experiment, we have investigated the characteristics of Singapore Marine Clay using the one-dimensional consolidation test.  The Virgin Curve and Unloading Curve of the sample was plotted and we found that the gradient of the Virgin Curve is 0.417 and is lower than theoretical values.  Reasons for the difference were discussed.  The water content of the sample had changed from 53.73% to 53.47% while the void ratio decreased from 1.289 to 1.077.  From the graph, we estimated the pre-consolidation pressure to be 71.5 kPa.  The shape of the graph indicates that once the soil is subject to a vertical loading pressure and is consolidated, removing the load will not allow the soil to go back to its original state as the soil is permanently consolidated.

In the permeability test, the permeability of coarse sand and marine clay were compared using two different methods - constant head test and falling head test.  Coarse sand has much larger permeability (about 1 million times) than clay.

# 6.	References

- Craig, R.F. (1992) Soil Mechanics, 5th edition, Chapman & Hall, London, 427.

- Karl Terzaghi (1966) Theoretical Soil Mechnics, John Wiley & Sons Inc, 479

- Nadal J. N. & Vivishikar D. G. (1995) Soil testing in Civil Engineering, A. A. Balkerna, USA, 249.

- T. William Lambe (1951) Soil testing for Engineering, John Wiley & Son, New York, 165.

# 7.	Appendix

## General Details of Specimen.

Soil:  			Singapore Marine Clay

Test Method: 	Consolidation Test using Odeometer

Assuming specific gravity of soil, Gs = 2.65 Mg/m3.

| Diameter of ring/soil (mm) | 70.0 |
| --- | --- |
| Height of ring/soil (mm), Ho | 19.0 |
| Weight of ring + wet soil (g) | 211.23 |
| Weight of ring only (g) | 83.66 |
| Weight of wet soil only (g) | 127.57 |
| Volume of soil (mm3) | 73120.57 |
| Bulk density, b (Mg/m3) | 1.745 |

| Specimen Test No. | I | II | III |
| --- | --- | --- | --- |
| Container + wet soil (g) | 31.24 | 47.74 | 38.80 |
| Container + dry soil (g) | 23.54 | 36.59 | 28.48 |
| Can only (g) | 9.21 | 15.77 | 9.18 |
| Water (g) | 7.70 | 11.15 | 10.32 |
| Dry Soil (g) | 14.33 | 20.82 | 19.30 |
| Moisture content, wo % | 53.73 | 53.55 | 53.47 |

Average moisture content, wo 		=	53.58%

Initial dry density, d = b / (1 + wo)	=	1.135 Mg/m3

Initial void ratio, eo = Gs / d – 1		=	1.333

Initial degree of saturation, So = woGs / eo =	106.54%

## 7.2	Compressibility.

Equivalent height of solid particles, Hs = Ho / (1 +eo) = 8.145 mm.

### 7.2.1	Void Ratio.

| Pressure (kPa) | Settlement (mm) | Cumu. Sett. ∆H (mm) | Consolidated Ht. H = Ho - ∆H (mm) | Vol. Strain s = ∆H/Ho | Void ratio e |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 19.00 | - | - |
| 25 | 0.3571 | 0.3571 | 18.64 | 0.01879 | 1.289 |
| 50 | 0.3409 | 0.6980 | 18.30 | 0.03674 | 1.247 |
| 100 | 0.6413 | 1.339 | 17.66 | 0.07049 | 1.168 |
| 200 | 1.226 | 2.565 | 16.43 | 0.1350 | 1.018 |
| 50 | -0.4789 | 2.086 | 16.91 | 0.1098 | 1.077 |

### 7.2.2	Compressibility.

| Pressure (kPa) | mv (m2/MN) |
| --- | --- |
| 25 | 0.7518 |
| 50 | 0.7314 |
| 100 | 0.7008 |
| 200 | 0.6940 |
| 50 | 0.1943 |

7.2.3	Values Taken from Void Ratio against Log Effective Stress Graph.

- Compression index, Cc	=	Slope of VCL
	=	0.24/0.1	=	0.417

- Swelling index, Cs	=	Slope of SWL
	=	0.08/0.5	=	0.160

- Pre-consolidation Pressure, Pc’	=	101.854	=	71.5 kPa

### 7.2.4	Coefficient of Consolidation (using square root time curve-fitting method and log time curve-fitting method).

| Pressure (kPa) | t90 (min) | t50 (min) | Hav = (H1 + H2)/2 (min) | Cv |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0.026 Hav2/t50 (m2/yr) | 0.111 Hav2/t90 (m2/yr) |
| 25 | 15.00 | 1.333 | 18.82 | 6.908 | 2.621 |
| 50 | 22.62 | 5.833 | 18.47 | 1.521 | 1.674 |
| 100 | 37.60 | 6.667 | 17.98 | 1.261 | 0.9544 |
| 200 | 50.42 | 11.67 | 17.05 | 0.6477 | 0.6399 |

### Permeability.

| Pressure (kPa) | Permeability kv = Cv mv w (cm/s) | Void ratio e |
| --- | --- | --- |
| 25 | 1.114 X 10-7 | 1.289 |
| 50 | 3.635 X 10-8 | 1.247 |
| 100 | 2.415 X 10-8 | 1.168 |
| 200 | 1.390 X 10-8 | 1.018 |
| 50 | - | 1.077 |

### 7.2.6	Final Water Content.

Final Void Ratio	=	1.077

H	=	16.91 mm

Volume of Water Lost	=	(19 - 16.91) x /4 x 702
	=	8043 mm3

Bulk Density	= 	Density of Water x Gs(1+wf)/(1+e)
{127.57 - 6221(10-3)} / (73120.57 - 6221) = 2.65(10-3)(1+wf)/(1+1.077)

Final Water Content, wf	=	42.17%

### 7.2.7	Summary of Results.

| Pressure (kPa) | Void ratio e | mv (m2/MN) | Cv (m2/yr) |  | kv (cm/s) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | t50 | t90 |  |
| 25 | 1.289 | 0.7518 | 6.908 | 2.621 | 1.114 X 10-7 |
| 50 | 1.247 | 0.7314 | 1.521 | 1.674 | 3.635 X 10-8 |
| 100 | 1.168 | 0.7008 | 1.261 | 0.9544 | 2.415 X 10-8 |
| 200 | 1.018 | 0.6940 | 0.6477 | 0.6399 | 1.390 X 10-8 |
| 50 | 1.077 | 0.1943 | - | - | - |

Using results from the G1 report on Singapore Marine Clay:

- Liquid Limit, LL		=	74.0%

- Plastic Limit, PL		=	30.3%

- Plasticity Index, PI	=	43.7%

    - Values of Given Correlations.

| Type of Soil | Formula | Value of Cc | Reference |
| --- | --- | --- | --- |
| Remoulded | 0.007 (LL – 10%) | 0.5915 | Terzaghi & Peck |
| Undisturbed | 0.009 (LL – 10%) | 0.7605 | Terzaghi & Peck |
| Remoulded, based on Critical State Concept | - 0.01346 PI - 0.02116 (PL – 9%) - 0.00828 (LL – 9%) | 0.7984 0.5540 0.7079 | Schofield & Wroth |
| Based on Critical State Concept | 0.05 Gs PI | 0.7860 | Wood |

## 7.3	Permeability of Soil.

#### 7.3.1	Constant Head Test.

Constant head test is conducted for coarse-grain soils where velocity of flow is generally fast.

Soil Description: 	Coarse sand

Head difference, h 	=	30.0 cm

Diameter of soil sample, D	=	7.5 cm

Area of soil sample, A	=	D2/4	=	44.18 cm2

Length of soil sample, L	=	10 cm

| Test No. | Time (s) | Discharge, Q (cm3) | kz = QL/Aht (cm/s) |
| --- | --- | --- | --- |
| 1 | 46.37 | 465 | 0.07566 |
| 2 | 69.66 | 690 | 0.07473 |
| 3 | 96.75 | 920 | 0.07174 |
| 4 | 102.72 | 1015 | 0.07455 |
| Average |  |  | 0.0742 |

#### 7.3.2	Falling Head Test.

The falling head test is conducted for fine-grained soils because the quantity of water passing through the soil is too small to be measured accurately under constant head test.

Soil Description: 	Singapore Marine Clay.

Diameter of standpipe, d	=	0.65 cm

Area of standpipe, a	=	d2/4		
=	0.332 cm2

Length of soil sample, L	=	8 cm

Diameter of soil sample, D	=	10 cm

Area of soil sample, A	=	D2/4		
=	78.54 cm2

| Test No. | Time, t (s) | h1 (cm) | h2 (cm) | kz = (aL/At) ln (h1/h2) (cm/s) |
| --- | --- | --- | --- | --- |
| 1 | 1800 | 138.3 | 134.1 | 5.794 X 10-7 |
| 2 | 3600 | 142.5 | 134.3 | 5.567 X 10-7 |
| 3 | 7200 | 143.2 | 127.1 | 5.602 X 10-7 |
| Average |  |  |  | 5.65 X 10-7 |