1	Abstract

In this experiment, four laboratory methods have been carried out to determine the undrained shear strength of a soil. They were the Undrained Triaxial Test (UU Test), the Laboratory Vane Shear Test, the Torvane Shear Test and the Cone Penetrometer Test. Shear strength values of cu are then obtained and compared with that calculated using the Wood’s Correlation. From the results obtained, it is clear that there is a tendency for the Wood’s Correlation to give an underestimation of the undrained shear strength.

# 2	Acknowledgements

I would like to thank Prof Nogami for his explanation on the experiment. Special thanks to Mr Ashish and Mdm Jamilah too, for providing us their vital assistance whenever it was required.

# 3	Objectives

3.1	To determine the undrained shear strength of soil using four laboratory methods.

3.2	To assess the measured shear strength values obtained from each method.

3.3	To compare the values obtained with the Wood’s correlation.

# 4	Equipment Used

4.1	Triaxial Test Set-up

4.2	Laboratory Vane Apparatus

4.3	Torvane Apparatus

4.4	Miniature Cone Penetrometer

# 5	Observations and Results

  - The soil sample tested in the experiment was the Singapore Marine Clay with the following Atterberg’s limits as obtained in Experiments G1 and G2:

Plastic Limit, PL 	= 30.3 %

Liquid Limit, LL 	= 74.0 %

Plasticity Index, PI 	= 43.7 %

  - The Wood’s Correlation is given by:

Cu = 170e(-4.6LI)

where LI is the liquidity index of the soil, LI = (w – PL)/PI

  - Below is a summary of the results obtained from the four tests conducted on the soil sample. Please refer to Appendix for further details.

|  | Undrained Shear Strength, Cu (kPa) | Cu from Wood’s Correlation (kPa) | Percentage Difference (%) |  |
| --- | --- | --- | --- | --- |
| Undrained Triaxial Test | 5 | 3.29 | 34.2 |  |
| Laboratory Vane Shear Test | 7.35 | 3.92 | 46.7 |  |
| Torvane Shear Test | 8.75 | 3.12 | 64.3 |  |
| Cone Penetrometer Test | 7.65 | 3.03 | 60.4 |  |

  - Figure 1 shows the shape of failure for the two tests conducted on the marine clay.

- Figure 1. Shape of failure

# 6	Discussions

  - Saturation procedure in the CIU Test

Firstly, apply a cell pressure (3) to the soil sample and observe the pore water pressure (U), then increase the cell pressure and observe the pore water pressure again. The changes in cell pressure and pore water pressure are then used to calculate the B factor, where B=U/3. Now, increase the back pressure, so that there is still a certain amount of effective stress, until it is equivalent to the pore water pressure. Then, increase the cell pressure and observe the pore water pressure again to obtain another value for B.

  - Why do the test results differ when different laboratory procedures are used?

    - Although the UU Test is one of the easiest and fastest tests available, it produces somewhat unreliable results. The effect of sampling, preparation, stress-relief and handling errors can produce unreliable values of cu. Besides, the in-situ undrained strength of clays is not isotropic and this will also result in different values of cu.

    - In the UU Test, the rubber membrane could have been excessively thick, or it could have been punctured to cause drainage during the experiment, as breakage of the membrane is hard to detect. A soiled cell base may also have caused the same effect. Poor water seals at ends or air bubbles in the pore water line may also be a source of error.

    - At the start of the UU Test, the test gauge only shows a reading of 100 when he cell pressure is already over 100. This shows that there is some error in the equipment used.

    - In the UU Test, the friction in the plunger may have contributed to the inaccuracies.

    - In the extrusion of the soil specimen, soil may have slipped out. The cutting edge of the sampling tube may not be good enough too.

    - When performing the Cone Penetrometer Test, eccentricity in loading can cause incorrect values for cu. Stones in the specimen may also affect the readings.

    - The Cone Penetrometer Test and Torvane Shear Test uses small equipment and thus they may be more prone to human errors than the other tests. In terms of accuracy, they are also worse than the UU Test since only simple apparatus are used. However, they are still preferred for on-site purposes for their convenience and ease in measurement.

    - In the Torvane Shear Test, the vane blades may not be totally seated into the soil surface.  When applying the torque manually, the torvane may not be standing at right angles to the soil surface. Such placement of the torvane at an inclined angle will result in a failure on a different plane.

    - In the Laboratory Vane Shear Test, the shaft could have been not vertical, or that the specimen was not placed horizontally, causing a failure in an inclined plane.

  - Why is the cell pressure calculated from the proving ring hanging free different from the cell pressure indicated from the pressure gauge?

In our experiment, we have assumed that k = 1.3428 N/div. This assumption may not be correct as it is based on the outdated data. The true present value may be very different from this assumed value.

6.4	Why are we getting a value of both ‘c’ and ‘’ in the Undrained Triaxial Test?

This could be due to the presence of air in the specimen. Given time, the air would dissolve in the pore water to result in a saturated sample. However, in our case, as the experiments were carried out in a short period of time, there wasn’t enough time for the air to dissolve totally. This resulted in an unsaturated soil sample that in turn caused  to arise.

# 7	Conclusion

The above tests conducted serve as a guide in estimating the shear strength parameters of soils. The Torvane Shear Test and the Cone Penetrometer Test may not provide very accurate results but they are able to provide engineers with a quick on-site estimate of the soil parameters. Also, the Wood’s Correlation tends to give an underestimation of the shear strength of a soil, but this method of estimation can be very convenient since a reasonable estimation may be made by simply using its Atterberg’s limits. All these estimations provide a range by which the shear strength may lie. Civil engineers are then able to design safe foundations according to the values obtained.

# 8	References

8.1	Lecture Notes

Professor Nogami

8.2	Basic Soil Mechanics, 3rd Edition

R. Whitlow, Longman

  - BS 1377: 1990: Methods of Test for Soils for Civil Engineering Purposes

British Standards Institution, London

8.4	Soil Mechanics, 6th Edition

Craig, R. F., E & FN Spon. London